Decision Models / B06.2350.01							Erik Grimshaw

A Guide to the Final Project 							Marcos March

December 1, 2003								Esteban Samboal

Jose Ignacio San Jose

# The Elevator Talk

The optimization model is built on a number of simplifying assumptions.  The most important of these are:

- Demand for KMC and Shimkin elevators occurs only on the first floor; inter-floor transit on the upper floors does not occur.

- Only peak time usage need be modeled.  These times occur at 9:00 am, 10:30 am, 12:00 pm, 1:30 pm, 3:00 pm, and 6:00 pm.

- All demand appears at once – arrival rates and their impact on elevator calls need not be modeled here (in fact, for the purposes of the model, there need not be control buttons inside the elevator; see below).

- Elevators are optimized to serve a certain set of floors for a certain number of runs from the ground floor to the highest floor to which it is assigned, and they will stop at every assigned floor on every run.

- No elevators are taken out of service for freight during peak times.

- Riders have no inherent preference for any elevator; for our purposes, all KMC and Shimkin elevators pick riders up from the same waiting area.

- Demand for each floor may be modeled by assuming a utilization rate of the office, classroom, computer lab, and conference room space on each floor.  Travel to lockers is also considered.

- People will prefer to walk up one flight of stairs rather than wait for an elevator.

- Riders will load themselves into each elevator such that the same number of people gets off at each floor the elevator serves (i.e., they load in fixed proportions that are dependent on the elevator).

Our model is constructed as follows:

- Binary 1/0 decision variables are introduced to indicate whether or not one of the seven elevators stops at each of the 11 floors.

- Additional decision variables determine the number of runs each elevator will make.

- The spreadsheet calculates the running time of each elevator per run based on the number of floors it is assigned to serve; these figures are multiplied by the number of runs assigned to track total runtime.  (Running time in transit between floors is assumed to be three seconds, and the time each elevator spends on a stop at one floor is assumed to be ten seconds.)

- The model is constrained to ensure that the total capacity served per each floor (across all seven elevators) meets or exceeds the demand given.  Each floor above the second floor is initially set to be served by at least three elevators, and each elevator is constrained to serve both the first floor and at least one additional floor (again, except second).

- The number of runs for the elevators is constrained such that the number of runs made by the busiest elevator does not exceed the number of runs made by the least active elevator by more than nine.

- The evolutionary solver is run to minimize the total runtime across all seven elevators.

Running the solver was itself an evolutionary process; after the initial run, where all floors were set to be served by at least three elevators, we identified which floors were being provided much more capacity than was needed, and reducing the service requirement for those floors – first to two elevators, and then to one if possible.  A similar process was initially followed with the balance constraint (the second to last in the list above), though this number was eventually left at nine since smaller numbers caused solver to run too slowly and consider fewer solutions each run than was ideal.  (And, after the balance constraint was settled, the need to initially constrain the service requirement abated, since its role, in part, was to balance the number of runs.)

The result is a set of optimized elevator/run configurations for the estimated demand for each peak time (the capacity constraint was adjusted each time to look to the demand row in question).  It is, obviously, a highly stylized solution to a highly stylized problem.  A model that accounted for fewer assumed properties may have been possible with an Extend model, but in either case we found that The Elevator Problem was likely more complex than we had the capability to model any more efficiently without conducting extensive research on alternative solution processes.