Conclusion and Recommendations:

To build a 30MW Solar PV plant in New York, the optimal cell type combination that can be used to minimize costs and maximize revenue is Polycrystalline Silicon. String Ribbon gives the least implementation cost but its efficiency was affected by random weather conditions, and thus was effective only 58% of the time.

Managerial Formulation

Decision Variables

Number of modules required from each module type to generate required amount of power in selected location (here, New York).

Objective

Select the PV module type that generates expected power and minimizes the total cost of the plant.

Constraints

For each cell type, plant capacity should be equal to the power the plant is claiming to produce.

The number of modules should be integers.

Mathematical Formulation

Decision Variables

Define Ni to be an integer variable identifying the number of modules of cell type i required in the plant.

Objective

Step 1:

Estimating Required Solar Electric (PV) System Size to Replace a Specified Amount of Utility (grid) electricity

PV System Capacity Required (kW of PV) can be roughly calculated as follows:

Annual electricity usage = Monthly Usage x 12 months (Electricity usage is express in kilowatt hours (kWh))

PV System Capacity Required = (Annual Usage ) / (78% x kWh/solar radiance(kW x 365 days)

Define

CNY =PV System Capacity Required to provide power for new York area.

AUNY = annual power usage in new york area

rNY =solar radiance in new york area(kWatt per Sq.meter)

CNY = AUNY / (78%* kWh / rNY *365)

Step 2:

We considered 5 types of  photovoltaic (PV) solar cells, with the following parameters:

For each cell type i,

Define

wi = Watts produced per square meter of solar panel area(w/sq.meter).

ai = Area of the solar panel (in sq.meters)

pi = total electric power produced by one module(Watts).

pi = wi * ai

Step 3:

Calculate the Solar Electric (PV) System output for each type of cell i used, i.e we need to convert Power(kW) to Energy (kWh), One kilowatt-hour (1 kWh) means an energy source supplies 1,000 watts (1 kW) of energy for one hour.

If the PV panels are shaded for part of the day, the output would be reduced in accordance to the shading percentage.

Define,

DHny = Yearly Averaged Daylight Hours (hours) in new york

CAny = yearly Averaged Daylight Cloud Amount (%) in new york

SHny = Number direct sun shine hours received in new york

SHny = DHny (1-CAny)

Iny = Averaged Insolation Incident On A Horizontal Surface (kWh/m2/day)

LCny = Land cost in new york.

For cell type I,

ei = Efficiency of solar panel for cell type i(%).

ci= Cost of each module of cell type i.

Mi = Module efficiency

Oi = Output of a PV panel system (kWh per year.)

Ni= Number of modules required for the plant if cell type i is used.

LCi = Cost of land for the total PV plant if we use cell type i.

ICi = Installation Cost of modules for the total PV plant if we use cell type i.

TCi= Transportation cost for modules

Ci = Cost for the total PV plant if we use cell type i.

LCi = Ni* ai*  LCny

ICi = Ni* ci

TCi = Shipping Weight of modules  of cell type i (lbs.) * Distance of production location of cell type i from NY(Miles) * Cost with Freight per mile per lb

Mi  = ei * pi * Iny

Oi = SHny * Mi

Ci = LCi +ICi + TCi

Our objective, now is to find the optimal type of cell (i) for which the cost is minimum :

Minimize C = minimum of ( i=15Ci),

Constraints

For all types of cells,

Ni is an integer.

Oi = CNY

Solution Methodology

Here’s the spreadsheet model:

Here are the Crystal-ball screenshots: