# Airplane Loading Simulation Model

Benjamin Macdonald

Jay Vin Lim

Patrick Geoghegan

Sulafa Bashir

### Problem:

- Problem: Airlines typically incur cost of $35-40 per minute of delay in turnaround time at gate

- Passenger loading time constitutes significant percentage of at-gate time

### Objective

- Simulate different passenger boarding strategies to identify most time-efficient

- Choice of Modeling Tool

  - Solver

  - Crystal Ball

  - Extend

- Simulation and Queuing properties of problem make Extend a natural choice

Model Parameters & Assumptions:

We define a plane 10 seats with, with 2 rows (3,4,3 seat configuration) with 60 rows. All rows have 10 seats (a simplifying assumption from real aircraft that narrow at the rear).

We assume all 600 seats have been sold.

Seat allocations are generated in the excel file Passenger_List_Generator.xls

There are a number of different worksheets for 20 row groups, 10 row groups, 5 row groups, and random passenger order.

Parameters generated:

| Row | Number from 1 to 60 |
| --- | --- |
| Seat | Number from 1 to 10 |
| Type | 1 = Business, 2 = Leisure, 3 = Family |
| Priority | Boarding order within a row. If we have priority across rows (window and middle of middle section boarding before aisle seats), then we have 3= highest priority, boarding first, 2 = middle priority, 1 = lowest priority (aisle). |
| GroupNbr | Passengers can be boarded in groups of 20, 10, or 5 rows. The highest priority group gets the highest group number. |

Passengers all have a unique row & seat number.

If we do random boarding, priority =1 & groupNbr = 1 for all passengers.

This spreadsheet should be refreshed (F9) and then the output fields listed above cut and pasted into a text document. This is the passenger manifest.

Extend Model:

Model Parts:

Passengers Arrive (Green Box):

The File object in extend within the Green box will read from the generated text file.

Attributes from the passenger manifest file are assigned to the passenger object (Row_Num, Seat_Num, passengerType, SeatPriority, BoardingGroup). The “Has_Baggage” attribute is also assigned at this point. This is assigned a 0 or 1 (1= has baggage), with a percentage chance of having baggage defined in the Baggage input box.

Attribute Generation (Yellow Box):

This box generates the time for passengers to board (walking time per row) and time to stow luggage if they have it. All are generated from log normal distributions, with a different mean and standard deviation for each passenger type.

Waiting Room – Load by Boarding Priority (Grey Blue Box):

The grey box contains a simulation of the waiting area. It has a sorted queue that orders passengers by BoardingGroup and SeatPriority. At the moment, this queue builds until 599 passengers have checked in. Once that many passengers have checked in and are in the waiting room queue, they will then board in order of BoardingGroup and SeatPriority.

The logical test can be changed  to wait for a smaller number of passengers – for example, the Airline may start boarding the plane after 400 people have checked in. This would result in the first 400 passengers boarding by priority group, then the remaining 200 passengers boarding as they turn up.

Which Aisle (Red Box):

This section looks at the seat number for passengers and directs them to Aisle 1 or Aisle 2. It performs a similar task to the Airline attendants at the entrance of the plane. From this, the model does a throw/catch to the first row of the relevant aisle.

Row 1 Aisle 1 (Blue Box):

Logic looks like:

Get into RowQueue

Get all attributes and look if we are in our row.

If we are not in our row, simply wait in queue until we can move ahead to next row.

If we are in our row, work out whether we are in the middle or on the window, then work out if there are people already sitting down in seats that will have to move for us to sit down. Then work out a delay time = time to stow baggage + time to move other passengers out of their seat if they are present + time for us to sit down. This time is then the delay for all other passengers in the aisle queue behind us.

If we have sat down, increment a global counter block, and send the time we sat down, and then reach an exit block.

Aisle 2 Initiation:

Aisle 2 has seat numbers 6 to 10. To replicate logic for aisle 1, the seat number for aisle 2 passengers is changed from X to 11-X (so it is then in the 1-5 range).

Remaining Rows and Aisles:

These are all identical logic to Row1 Aisle 1, but with different row numbers. They use a custom defined cell that contains all the logic of Row 1 Aisle 1.

Running the model:

The first step is to ensure the correct data input file is selected in the file block of “Passengers Arrive”. Then run the simulation. It is a lot faster without animation (taking about 40 seconds on lab computers per run versus many minutes with animation). At the bottom left of the model is the summary counter block. This shows the time each passenger sat down. When the simulation is complete, the time for the last passenger to sit can be obtained from this block.

The Model can be run with same passenger manifest several times as walk time, has_baggage and time to stow baggage variables are randomly generated.

Possible Extensions to the Model

This model could be extended to include:

- Priority boarding of frequent fliers or small children out of priority sequence

- People who go down wrong aisle

- Planes with <600 seats sold

- Multiple door boarding (front and rear door) simultaneously.

The model was run with data:

Random, 5 row groups, 10 row groups , 20 row groups, 10 row groups with window priority, 20 row groups with window priority. We generated 10 manifests for each alternative, and ran each manifest 5 times – 500 total runs of the application. Results are available in the powerpoint presentation.