Solutions to SPC Practice Problems

1. The overall average on a process you are attempting to monitor is 50 units. The process standard deviation is known to be 1.72. Determine the upper and lower control limits for a mean chart, if you choose to use a sample size of 5. Set z = 3.

The control limits are calculated as follows:

| Or | (47.69, 52.31) |
| --- | --- |

2. Food Storage Technologies produces refrigeration units for food producers and retail food establishments. The overall average temperature that these units maintain is 46 Fahrenheit. The average range is 2 Fahrenheit. Samples of 6 are taken to monitor the production process. Determine the upper and lower control limits for both a mean chart and a range chart for these refrigeration units.

Since the standard deviation is not known, we will use the given information about the average range of the samples, together with the following table:

| Sample Size, n | Mean Factor, A2 | Upper Range, D4 | Lower Range, D3 |
| --- | --- | --- | --- |
| 2 | 1.880 | 3.268 | 0 |
| 3 | 1.023 | 2.574 | 0 |
| 4 | 0.729 | 2.282 | 0 |
| 5 | 0.577 | 2.115 | 0 |
| 6 | 0.483 | 2.004 | 0 |
| 7 | 0.419 | 1.924 | 0.076 |
| 8 | 0.373 | 1.864 | 0.136 |
| 9 | 0.337 | 1.816 | 0.184 |
| 10 | 0.308 | 1.777 | 0.223 |
| 12 | 0.266 | 1.716 | 0.284 |

Factors for Computing 3 Sigma Control Chart Limits

For the mean chart:

| Or | (45.03, 46.97) |
| --- | --- |

For the range chart:

3. Sampling 4 pieces of precision-cut wire (to be used in computer assembly) every hour for the past 24 hours has produced the following results:

| Hour | (inches) | R (inches) | Hour | (inches) | R (inches) |
| --- | --- | --- | --- | --- | --- |
| 1 | 3.25 | 0.71 | 13 | 3.11 | 0.85 |
| 2 | 3.10 | 1.18 | 14 | 2.83 | 1.31 |
| 3 | 3.22 | 1.43 | 15 | 3.12 | 1.06 |
| 4 | 3.39 | 1.26 | 16 | 2.84 | 0.50 |
| 5 | 3.07 | 1.17 | 17 | 2.86 | 1.43 |
| 6 | 2.86 | 0.32 | 18 | 2.74 | 1.29 |
| 7 | 3.05 | 0.53 | 19 | 3.41 | 1.61 |
| 8 | 2.65 | 1.13 | 20 | 2.89 | 1.09 |
| 9 | 3.02 | 0.71 | 21 | 2.65 | 1.08 |
| 10 | 2.85 | 1.33 | 22 | 3.28 | 0.46 |
| 11 | 2.83 | 1.17 | 23 | 2.94 | 1.58 |
| 12 | 2.97 | 0.40 | 24 | 2.64 | 0.97 |

Develop appropriate control charts and determine whether there is any cause for concern in the cutting process. Plot the information and look for patterns.

First, we’ll compute the upper and lower limits for both an  chart and an R chart. For the mean chart, note that  and :

| Or | (2.236, 3.728) |
| --- | --- |

For the range chart:

Now, we plot the charts:

Based on our rules for interpreting control charts, this process can be assumed to be in control.

4. Small boxes of NutraFlakes cereal are labeled “net weight 10 ounces.” Each hour, random samples of size n = 4 boxes are weighed to check process control. Five hours of observations yielded the following data:

|  | Weights |  |  |  |
| --- | --- | --- | --- | --- |
| Time | Box 1 | Box 2 | Box 3 | Box 4 |
| 9 a.m. | 9.8 | 10.4 | 9.9 | 10.3 |
| 10 a.m. | 10.1 | 10.2 | 9.9 | 9.8 |
| 11 a.m. | 9.9 | 10.5 | 10.3 | 10.1 |
| 12 a.m. | 9.7 | 9.8 | 10.3 | 10.2 |
| 1 p.m. | 9.7 | 10.1 | 9.9 | 9.9 |

- Using these data, construct limits for  and R charts.

We note that  and . For the mean chart:

| Or | (9.66, 10.42) |
| --- | --- |

For the range chart:

- Is the process in control?

The process appears to be in control.

- What other steps should the quality department follow at this point?

There aren’t really very much data here, nor are the sample sizes very large. We could increase the sample size and continue to monitor the charts over time.

5. You are attempting to develop a quality monitoring system for some parts purchased from Warton & Kotha Manufacturing Co. These parts are either good or defective. You have decided to take a sample of 100 units. Develop a table of the appropriate upper and lower control chart limits for various values of the fraction defective in the sample taken. The values for p in this table should range from 0.02 to 0.10 in increments of 0.02. Develop the upper and lower control limits for a 99.73% confidence interval.

Here is how to calculate the limits for the first value of the proportion defective:

| Or | (0.0000, 0.0620) |
| --- | --- |

(Note that we have substituted zero for any negative number. In this case the lower control limit would have been –0.022.)

Here is the completed table of control limits:

|  | n = 100 |  |
| --- | --- | --- |
| p | UCL | LCL |
| 0.02 | 0.0620 | 0.0000 |
| 0.04 | 0.0988 | 0.0000 |
| 0.06 | 0.1312 | 0.0000 |
| 0.08 | 0.1614 | 0.0000 |
| 0.10 | 0.1900 | 0.0100 |

6. In the past, the defect rate for your product has been 1.5%. What are the upper and lower control chart limits if you wish to use a sample size of 500 and z = 3?

| Or | (0.000, 0.0313) |
| --- | --- |

7. Refer to problem 6 above. If the defect rate were 3.5% instead of 1.5%, what would be the control limits?

| Or | (0.0103, 0.0597) |
| --- | --- |

8. Refer to problems 6 and 7 above. Management would like to reduce the sample size to 100 units. If the past defect rate has been 3.5%, what would happen to the control limits (z = 3)? Should this action be taken? Explain your answer.

| Or | (0.0000, 0.0901) |
| --- | --- |

This turns out to result in a rather large increase in the width of the control range (the upper limit increases by more than 50%). This may have a significant negative effect on quality, by increasing the risk of a Type II error.

9. Blackburn, Inc., an equipment manufacturer in Nashville, has submitted a sample cutoff valve to improve your manufacturing process. Your process engineering department has conducted experiments and found that the valve has a mean () of 8.00 and a standard deviation () of 0.04. Your desired performance is defined by the specification limits (7.865, 8.135). What is the Cpk of the Blackburn valve?

This process is capable. It is centered between the specification limits, and its variability is sufficiently small that it will meet customer needs almost all of the time.

10. The manager of the Oat Flakes plant desires a quality specification with a mean of 16 ounces, an upper specification limit of 16.5, and a lower specification limit of 15.5. In this example, the process is known to have a population standard deviation of 1.0 ounces. Using the data below, in which weights are expressed in ounces, determine the standard deviation of the 12 weights and then determine the Cpk of the process.

| Weight of Sample |  | Weight of Sample |  | Weight of Sample |  |
| --- | --- | --- | --- | --- | --- |
| Hour | Avg. of 9 Boxes | Hour | Avg. of 9 Boxes | Hour | Avg. of 9 Boxes |
| 1 | 16.1 | 5 | 16.5 | 9 | 16.3 |
| 2 | 16.8 | 6 | 16.4 | 10 | 14.8 |
| 3 | 15.5 | 7 | 15.2 | 11 | 14.2 |
| 4 | 16.5 | 8 | 16.4 | 12 | 17.3 |

Note that .

This process is not capable; it is too variable to meet customer needs.

Table S6.1 from Heizer and Render, p. 204 (Factors for Computing 3 Sigma Control Chart Limits)

Similar to Exhibit TN7.7, p. 310, in Chase, Jacobs, Aquilano.